## Lab Task Report: Parallel DC Circuit Simulation

## TASK 1

### 1. Software Initialization and Component Selection

The process began by launching Proteus 8 Professional and creating a new schematic capture. Using the "Pick Devices" (P) feature, the following essential components were selected from the library:

- Battery (VSOURCE): Set to a constant 9V DC.

- Resistors (RES): Three resistors with values 10kΩ R1, 2kΩ (R2), and 1kΩ (R3).

- Measuring Instruments: Three DC Ammeters to measure branch currents and one DC Voltmeter for potential difference.

### 2. Circuit Construction (Schematic Design)

The circuit was designed in a Parallel Configuration.

- The positive terminal of the 9V battery was connected to a common top rail (Node 1-2-3-4).

- Each resistor was placed in its own vertical branch.

- The negative terminal was connected to a common bottom rail (Node 5-6-7-8) and tied to a Ground (GND) terminal to provide a 0V reference point for the simulation engine.

### 3. Instrument Integration and Configuration

To ensure accurate data collection, the instruments were placed as follows:

- Ammeters: Integrated in series within each resistor's branch. The "Display Range" was specifically modified from Amps to Milliamps (mA) because the high resistance values k ohm result in small current flows.

- Voltmeter: Connected in parallel between Node 2 and Node 7. This allows the measurement of the voltage drop across the parallel network.

### 4. Simulation and Troubleshooting

During the initial run, several technical issues were identified and resolved:

- Polarity Check: Ammeters were flipped to ensure the positive (+) terminal faced the conventional current flow.

- Short Circuit Removal: An accidental bypass wire between nodes was removed to allow current to flow through the resistors instead of around them.

- Node Integrity: Ensured all junctions were properly soldered (dotted) in the software to avoid "Open Circuit" errors.

### 5. Final Results and Verification

Once the "Play" button was pressed, the simulation reached a steady state. The measured (Practical) values perfectly matched the calculated (Theoretical) values derived from Ohm’s Law (I = V / R):

- For R1(10kΩ): 9V / 10,000OHM= 0.90mA

- For R2 (2kΩ): 9V / 2,000OHM = 4.50mA

- For R3 (1kΩ): 9V / 1,000OHM = 9.00mA

- Node 2-7 Voltage: Remained constant at 9.00V, confirming the fundamental rule of parallel circuits.

Conclusion: The experiment successfully demonstrates that in a parallel circuit, voltage remains constant across all branches, while the current divides based on the individual resistance of each path.

## Lab Task 2 Report: Multi-Branch Parallel Circuit Simulation

### 1. Procedure & Software Setup

The experiment began by setting up a new schematic in Proteus 8 Professional. To match the requirements of Task 2, the following components were placed on the workspace:

- DC Voltage Source: A battery set to 9V.

- Resistor Network: Four resistors with values 100OHM R1, 3k OHM (R2), 5k OHM(R3), and 10k OHM (R4).

- Measurement Tools: Four DC Ammeters and one DC Voltmeter.

### 2. Circuit Construction & Troubleshooting

The resistors were connected in a Parallel Configuration between two main nodes (Node 'a' and Node 'e'). During the construction, several technical adjustments were made:

- Polarity Correction: Initially, the meters showed negative values. The battery orientation was corrected (Positive terminal to the left) to ensure conventional current flow and positive readings.

- Range Optimization: Since the resistance values ranged from 100 OHM to 10k OHM, the Ammeter for R1 was kept in Amps (A), while the meters for R2, R3 and R4 were switched to Milliamps (mA) to display precise decimal values.

### 3. Theoretical Calculations (Ohm's Law Verification)

To verify the simulation, theoretical values were calculated using the formula I = V / R:

- For R1 (100 OHM): I = 9V / 100ohm = 0.09A

- For R2 (3k OHM): I = 9V / 3000 OHM = 3.00mA

- For R3 (5k OHM): I = 9V / 5000 OHM = 1.80mA

- For R4 (10k OHM): I = 9V / 10000 OHM = 0.90mA

### 4. Experimental Results & Observations

Upon running the simulation, the practical readings on the Proteus meters perfectly matched the theoretical calculations:

- Voltage Stability: The DC Voltmeter connected across nodes 'a' and 'e' showed a constant 9.00V, proving that voltage does not divide in a parallel circuit.

- Current Relationship: It was observed that the 100OHM resistor (lowest resistance) allowed the highest current flow (0.09A), confirming that Current is inversely proportional to Resistance.

Conclusion: The simulation successfully validated the characteristics of a parallel DC circuit. The results confirm that while voltage remains constant across all parallel branches, the current divides according to the specific resistance of each path.