## Lab Report: Design and Simulation of an Inverting Amplifier

### 1. Objective

The objective of this lab is to design an Inverting Amplifier circuit using the LM741 Operational Amplifier (Op-Amp) and to analyze the relationship between input and output waveforms using Proteus simulation software.

### 2. Apparatus and Components

The following components were selected from the Proteus library:

- Operational Amplifier: LM741 IC.

- Resistors: $R_1 = 1k\Omega$ (Input Resistor) and $R_2 = 2k\Omega$ (Feedback Resistor).

- Input Source: VSINE (AC Sine Wave Generator) set to $40Hz$ frequency.

- Measuring Instrument: Digital Oscilloscope.

- Power Supplies: DC Voltage sources ($+12V$ for $V_{CC}$ and $-12V$ for $V_{EE}$).

- Ground: Common reference points.

### 3. Procedure

- Component Placement: The LM741 Op-Amp was placed at the center of the schematic capture.

- Input Circuitry: The Sine Wave generator ($V_1$) was connected to the Inverting Terminal (Pin 2) through the input resistor $R_1$ ($1k\Omega$).

- Feedback Loop: The feedback resistor $R_2$ ($2k\Omega$) was connected between the Inverting Terminal (Pin 2) and the Output Terminal (Pin 6).

- Reference Connection: The Non-Inverting Terminal (Pin 3) was connected directly to Ground.

- Biasing: Pin 7 was connected to a $+12V$ DC source, and Pin 4 was connected to a $-12V$ DC source to power the IC.

- Measurement Setup:

  - Channel A of the oscilloscope was connected to the input signal (before $R_1$).

  - Channel B was connected to the output at Pin 6.

- Simulation: The "Play" button was pressed, and the Digital Oscilloscope window was opened via the Debug menu to observe the waveforms.

### 4. Calculations

The theoretical Voltage Gain ($A_v$) for an inverting amplifier is calculated as:

$$A_v = -\frac{R_2}{R_1}$$

Using the values from the circuit:

$$A_v = -\frac{2k\Omega}{1k\Omega} = -2$$

This indicates that the output signal will be twice the size of the input signal and will be inverted ($180^{\circ}$ out of phase).

### 5. Results and Observations

- Amplification: The Blue trace (Output) on the oscilloscope appeared twice as large as the Yellow trace (Input).

- Phase Inversion: It was observed that when the input sine wave reached its positive peak, the output sine wave simultaneously reached its negative peak.

- Stability: The output remained stable within the $\pm 12V$ saturation limits provided by the power supplies.

### 6. Conclusion

The simulation successfully demonstrated the characteristics of an Inverting Amplifier. The calculated gain of $-2$ matched the visual results on the oscilloscope. The project confirms that the gain of such an amplifier is solely determined by the ratio of the external resistors $R_1$ and $R_2$.

Here is the English lab report specifically for the circuit shown in WhatsApp Image 2026-05-03 at 10.52.30 AM (1).jpeg. This version uses the $3k\Omega$ resistor value visible in your latest image.

## Lab Report: Simulation of an Inverting Amplifier (Gain of 3)

### 1. Objective

The purpose of this experiment is to design and simulate an Inverting Amplifier using the LM741 Operational Amplifier in Proteus and to verify the voltage gain and phase inversion through oscilloscope observations.

### 2. Components and Software

- Software: Proteus 8 Professional.

- Active Device: LM741 Operational Amplifier.

- Resistors: $R_1 = 1k\Omega$ (Input) and $R_2 = 3k\Omega$ (Feedback).

- Signal Source: VSINE (AC Source).

- Power Supplies: DC Sources ($\pm 12V$ or $\pm 15V$ for biasing).

- Output Device: Digital Oscilloscope.

### 3. Procedure

- Placement: The LM741 Op-Amp was placed on the schematic.

- Input Path: An AC sine wave generator ($V_1$) was connected to the Inverting Terminal (Pin 2) via the input resistor $R_1$ ($1k\Omega$).

- Feedback Network: A feedback resistor $R_2$ ($3k\Omega$) was connected between the output (Pin 6) and the inverting input (Pin 2).

- Grounding: The Non-Inverting Terminal (Pin 3) was tied to Ground to maintain the inverting configuration.

- Biasing: Proper DC voltages were applied to Pin 7 ($+V_{CC}$) and Pin 4 ($-V_{EE}$).

- Oscilloscope Connections:

  - Channel A was connected to the input signal source.

  - Channel B was connected to the output at Pin 6.

- Execution: The simulation was started, and the waveforms were analyzed on the Digital Oscilloscope.

### 4. Mathematical Analysis

The theoretical gain ($A_v$) of the circuit is determined by the ratio of the feedback resistor to the input resistor:

$$A_v = -\frac{R_2}{R_1}$$

Substituting the values from the image:

$$A_v = -\frac{3k\Omega}{1k\Omega} = -3$$

Conclusion from Calculation: The output signal should be three times larger than the input signal and shifted by 180 degrees.

### 5. Observations

- Amplitude: The output waveform (Channel B) showed an amplitude three times greater than the input waveform (Channel A).

- Phase: A clear phase reversal was observed; when the input reached its maximum positive value, the output reached its maximum negative value.

- Integrity: The output wave was a clean sine wave, confirming that the Op-Amp was operating within its linear region.

### 6. Conclusion

The lab successfully demonstrated the operation of an inverting amplifier. The practical results obtained from the Proteus simulation matched the theoretical calculation of a gain of $-3$. This confirms that the gain of an inverting amplifier can be precisely controlled by selecting appropriate resistor values.