# Lab Report

## Experiment: Verification of Norton’s Theorem

### Objective

To verify Norton's Theorem by finding Norton current (IN)(I_N)(IN​), Norton resistance (RN)(R_N)(RN​), and comparing the load current in the original circuit and Norton equivalent circuit.

# Apparatus Required

- Regulated Power Supply (0–30 V)

- Resistors (330 Ω, 1 kΩ)

- Ammeter (0–30 mA)

- Breadboard / Circuit board

- Connecting wires

- Digital Multimeter (DMM)

- Proteus Software

# Theory

Norton’s Theorem states that any linear bilateral network can be replaced by an equivalent circuit consisting of a current source in parallel with an equivalent resistance.

The equivalent current is called Norton current (IN)(I_N)(IN​) and the equivalent resistance is called Norton resistance (RN)(R_N)(RN​).

# Circuit Components

- R1=330 ΩR_1 = 330 \, \OmegaR1​=330Ω

- R2=330 ΩR_2 = 330 \, \OmegaR2​=330Ω

- R3=330 ΩR_3 = 330 \, \OmegaR3​=330Ω

- Load Resistance RL=1 kΩR_L = 1 \, k\OmegaRL​=1kΩ

- Supply Voltage E=30 VE = 30 \, VE=30V

# Procedure

## Step 1: Original Circuit

- Connect the circuit according to the given diagram in Proteus.

- Set the DC supply voltage to 30 V.

- Connect the load resistor RL=1kΩR_L = 1k\OmegaRL​=1kΩ.

- Measure and note the load current ILI_LIL​.

## Step 2: Finding Norton Current (IN)(I_N)(IN​)

- Remove the load resistance RLR_LRL​.

- Short-circuit the load terminals.

- Run the simulation.

- Measure the short-circuit current using the ammeter.

- This current is Norton current (IN)(I_N)(IN​).

Measured Value:

IN=22.50 mAI_N=22.50\,mAIN​=22.50mA

## Step 3: Finding Norton Resistance (RN)(R_N)(RN​)

- Remove the load resistor and turn OFF the voltage source.

- Find the equivalent resistance across the load terminals.

Calculation:

RN=R2+(R1∥R3)R_N=R_2+(R_1\parallel R_3)RN​=R2​+(R1​∥R3​)

RN=330+330×330330+330R_N = 330 + \frac{330 \times 330}{330 + 330}RN​=330+330+330330×330​ RN=330+165=495 ΩR_N = 330 + 165 = 495 \, \OmegaRN​=330+165=495Ω

## Step 4: Norton Equivalent Circuit

- Replace the original network with:

  - Norton current source INI_NIN​

  - Norton resistance RNR_NRN​ in parallel

- Connect the load resistor RLR_LRL​.

- Measure the load current again.

Measured Load Current:

IL=15.00 mAI_L=15.00\,mAIL​=15.00mA

# Observation Table

| E (volts) | IN (mA) | RN (Ω) | IL (mA) Circuit-I | IL (mA) Equivalent Circuit |
| --- | --- | --- | --- | --- |
| 30 V | 22.50 mA | 495 Ω | 15.00 mA | 15.00 mA |

# Calculations

## Norton Voltage

VN=IN×RNV_N=I_N\times R_NVN​=IN​×RN​

VsV_sVs​

V

RRR

Ω\OmegaΩ

I=VsR=12.0 V6.0 Ω=2.00 AI = \frac{V_s}{R} = \frac{12.0\,\mathrm{V}}{6.0\,\Omega} = 2.00\,\mathrm{A}I=RVs​​=6.0Ω12.0V​=2.00A

Vs = 12.0 V+-R = 6.0 ΩI = 2.00 A

VN=22.50×10−3×495V_N = 22.50 \times 10^{-3} \times 495VN​=22.50×10−3×495 VN=11.14 VV_N = 11.14 \, VVN​=11.14V

## Percentage Error

% Error=∣IL(original)−IL(Norton)∣IL(original)×100\%\,Error=\frac{|I_{L(original)}-I_{L(Norton)}|}{I_{L(original)}}\times100%Error=IL(original)​∣IL(original)​−IL(Norton)​∣​×100

%Error=∣15−15∣15×100\% Error = \frac{|15-15|}{15}\times100%Error=15∣15−15∣​×100 %Error=0%\% Error = 0\%%Error=0%

# Result

The load current in the original circuit and Norton equivalent circuit is approximately equal.

Therefore, Norton’s Theorem is verified successfully.

# Precautions

- Ensure proper circuit connections before simulation.

- Use correct resistor values.

- Set the ammeter range properly.

- Remove the load resistor carefully while finding INI_NIN​.

- Turn OFF the voltage source while calculating RNR_NRN​.