# Verification of Kirchhoff’s Current Law (KCL) using Proteus

## Objective

To verify Kirchhoff’s Current Law using simulation in Proteus software by measuring currents at a node and confirming that the sum of incoming current is equal to the sum of outgoing currents.

## Apparatus / Components Used

The following components were used in Proteus ISIS:

- DC Power Supply (RPS)

- Resistors:

  - 330Ω

  - 220Ω

  - 1kΩ (two resistors)

- DC Ammeters (0–30 mA) × 3

- Ground terminal

- Connecting wires

## Theory

According to Kirchhoff’s Current Law,
the algebraic sum of currents at any node in a circuit is zero.

Mathematically:

I1=I2+I3I_1 = I_2 + I_3I1​=I2​+I3​

Where:

- I1I_1I1​ = Incoming current

- I2,I3I_2, I_3I2​,I3​ = Outgoing currents

## Procedure

- Opened Proteus ISIS and created a new project.

- Selected required components using the “Pick Devices” option.

- Placed all components on the workspace:

  - One DC voltage source

  - Four resistors

  - Three ammeters

  - Ground terminal

- Connected the circuit according to the given KCL diagram:

  - 330Ω resistor connected in series with Ammeter A1

  - From the node, two branches were formed:

    - One branch with Ammeter A2 and 1kΩ resistor (current I2I_2I2​)

    - Second branch with Ammeter A3, 220Ω resistor, and 1kΩ resistor (current I3I_3I3​)

- Connected the negative terminal of the supply to ground.

- Set resistor values:

  - R1 = 330Ω

  - R2 = 220Ω

  - R3 = 1kΩ

  - R4 = 1kΩ

- Set DC supply voltage to different values:

  - 5V

  - 15V

  - 25V

- Ran the simulation using the “Run” (Play) button.

- Recorded current readings from ammeters:

  - A1 → I1I_1I1​

  - A2 → I2I_2I2​

  - A3 → I3I_3I3​

- Verified that I1=I2+I3I_1 = I_2 + I_3I1​=I2​+I3​ for each case.

## Observations

| S. No | Voltage (V) | I₁ (mA) | I₂ (mA) | I₃ (mA) | I₁ = I₂ + I₃ |
| --- | --- | --- | --- | --- | --- |
| 1 | 5 | 5.68 | 3.12 | 2.56 | Verified |
| 2 | 15 | 17.05 | 9.36 | 7.69 | Verified |
| 3 | 25 | 28.40 | 15.60 | 12.80 | Verified |

## Result

​ Hence, Kirchhoff’s Current Law is verified.

## Conclusion

The simulation results confirm that the current entering a node is equal to the current leaving the node. This validates Kirchhoff’s Current Law practically using Proteus.

## Precautions

- Ammeter should always be connected in series.

- Proper grounding must be ensured.

- Correct values of resistors should be used.

- Connections must be checked before running the simulation.

# Verification of Kirchhoff’s Voltage Law (KVL) Using Proteus

## Objective

To verify Kirchhoff’s Voltage Law using Proteus ISIS by measuring voltage drops across resistors in a closed loop and confirming that the total supplied voltage is equal to the sum of all voltage drops.

## Apparatus / Components Used

The following components were used in the simulation:

- DC Voltage Source (RPS / BAT1)

- Resistors:

  - 330Ω

  - 220Ω

  - 1kΩ

- DC Voltmeters (0–30V) × 3

- Ground terminal

- Connecting wires

## Theory

According to Kirchhoff’s Voltage Law:

In any closed loop of an electrical circuit, the algebraic sum of all voltages is zero.

Mathematically:

E1=V1+V2+V3E1 = V1 + V2 + V3E1=V1+V2+V3

Where:

- E1 = Supply voltage

- V1, V2, V3 = Voltage drops across resistors

## Circuit Description

The circuit consists of a DC voltage source connected with three resistors (330Ω, 220Ω, and 1kΩ). Voltmeters are connected in parallel across each resistor to measure voltage drops.

## Procedure

- Opened Proteus ISIS and created a new schematic.

- Selected required components using “Pick Devices”.

- Placed the following components:

  - DC Voltage Source (BAT1)

  - Resistors (330Ω, 220Ω, 1kΩ)

  - Three DC Voltmeters

  - Ground terminal

- Connected the circuit according to KVL diagram:

  - Resistors connected in series loop

  - Voltmeters connected in parallel across each resistor

- Set the voltage source to different values:

  - 5V

  - 15V

  - 25V

- Ran the simulation.

- Observed and recorded voltage readings from V1, V2, and V3.

- Verified KVL equation for each case.

## Observations

### 🔹 At 5V:

- V1 = 2.62 V

- V2 = 1.77 V

- V3 = 0.61 V
✔ Sum = 5.00 V

### 🔹 At 15V:

- V1 = 7.86 V

- V2 = 5.31 V

- V3 = 1.83 V
✔ Sum = 15.00 V

### 🔹 At 25V:

- V1 = 13.10 V

- V2 = 8.85 V

- V3 = 3.05 V
✔ Sum = 25.00 V

## Result

The simulation confirms that:

E1=V1+V2+V3E1 = V1 + V2 + V3E1=V1+V2+V3

Thus, Kirchhoff’s Voltage Law is successfully verified using Proteus ISIS.

## Conclusion

The experiment shows that the total supplied voltage in a closed loop is equal to the sum of voltage drops across all resistors. Hence, Kirchhoff’s Voltage Law is verified practically.

## Precautions

- Voltmeters must be connected in parallel.

- Correct resistor values must be ensured.

- Proper ground connection is required.

- Simulation must be run after checking all connections.