Circuit Analysis (Lab10)

Experiment 9: Verification of Thevenin’s Theorem

RABEEA IMTIAZ	2025 CE 75

Name: …………………………….	 Registration No: ………………………... Date: …………………………….	Grade and Signature: ……………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeter, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  |  |  |
|  | Total marks (10) |  |  |  |

# Objective

The objective of this lab is verify Thevenin’s theorem and to find the full load current.

# Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k, 330 ohms | 1,3 |
| 7 | Potentiometer | Standard | 1 |

Description

## Statement:

Any linear bilateral, active two terminal network can be replaced by an equivalent voltage source (VTH). Thevenin’s voltage or V OC in series with looking pack resistance RTH.

## Precautions:

- Voltage control knob of RPS should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position

## Procedure:

- Connections are given as per the circuit diagram.

- Set a particular value of voltage using RPS and note down the corresponding ammeter readings. To find VTH 3. Remove the load resistance and measure the open circuit voltage using multimeter (VTH). To find RTH

- To find the Thevenin’s resistance, remove the RPS and short circuit it and find the RTH using multimeter.

- Give the connections for equivalent circuit and set VTH and RTH and note the corresponding ammeter reading. 6. Verify Thevenins theorem.

Circuit - 1: To find load current

To find VTH

To find RTH

Thevenin’s Equivalent circuit:

Thevenin Practical Values

|  | E(V) | VTH(V) | RTH(ohms) | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit - I | Equivalent Circuit |
| Values | 10 | 5 | 495 | 3.34 | 3.33 |

# Post Lab Questions

What is Thevenin’s Theorem?

states that any linear electrical circuit containing voltage sources, current sources, and resistances can be replaced by a simple equivalent circuit consisting of:

- a single equivalent voltage source (VTH)

- in series with a single equivalent resistance (RTH)

👉 This simplifies complex circuits into an easier form for analysis, especially for finding load current and load voltage

What are the applications of Thevenin’s Theorem?

Thevenin’s theorem is commonly used for:

- 🔹 Simplifying complex electrical circuits

- 🔹 Calculating load current and load voltage easily

- 🔹 Electronic circuit analysis (amplifiers and networks)

- 🔹 Power system analysis and equivalent modeling

- 🔹 Maximum power transfer studies

- 🔹 Fault analysis in electrical systems

What are the theoretical and practical limitations of Thevenin’s Theorem?

### 📘 Theoretical Limitations:

- It applies only to linear circuits

- Not valid for non-linear components (e.g., diodes, transistors in active region)

- Limited use in time-varying or dynamic systems

### 🔧 Practical Limitations:

- Finding VTH and RTH can be time-consuming in large circuits

- Requires accurate measurements for correct results

- Small errors may occur in practical lab or simulation setups (e.g., Proteus)

- Dependent sources make analysis more complex