Circuit Analysis (Lab11)

Experiment 10: Verification of Norton’s Theorems

RABBEA IMTIAZ	2025 CE 75

Name: …………………………….	 Registration No: ………………………... Date: …………………………….	Grade and Signature: ……………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeter, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  |  |  |
|  | Total marks (10) |  |  |  |

# Objective

The objective of this lab is verify Nortan’s theorem and to find the full load current.

# Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k, 330 ohms | 1,3 |
| 7 | Potentiometer | Standard | 1 |

Description

Statement:

Any linear circuit can be simplified to an equivalent circuit consisting of a single current source and parallel resistance that is connected to a load.

Precautions:

- Voltage control knob of RPS should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position

# Procedure for Norton’s Theorem:

- Connections are given as per circuit diagram.

- Set a particular value in RPS and note down the ammeter readings in the original circuit.

To Find IN:

- Remove the load resistance and short circuit the terminals.

- For the same RPS voltage note down the ammeter readings.

To Find RN:

- Remove RPS and short circuit the terminal and remove the load and note down the resistance across the two terminals.

Equivalent Circuit:

- Set IN and RN and note down the ammeter readings.

- Verify Norton’s theorem.

To find load current in circuit 1:

To find IN

To find RN

# Norton’s equivalent circuit

Norton’s Practical Values

|  | E (volts) | IN (mA) | Rn (ohms) | IL (mA) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Circuit - I | Equivalent Circuit |
| Practical Values | 30 22.50mA |  | 495ohm | 15mA | 15mA |

# POST Lab Questions

- What is Nortan’s Theorem?

Norton's Theorem states that any linear bilateral electrical network can be replaced by an equivalent circuit consisting of a current source in parallel with an equivalent resistance.
The equivalent current is called Norton current (IN)(I_N)(IN​) and the equivalent resistance is called Norton resistance (RN)(R_N)(RN​).
This theorem is used to simplify complex circuits for easy analysis.

- What are the applications of Nortan’s Theorem?

Applications of Norton’s Theorem include:

- Simplifying complex electrical circuits

- Calculating load current and voltage easily

- Used in circuit analysis and troubleshooting

- Helpful in electronic and power system design

- Used for studying different load conditions

- Used in network analysis and electrical engineering calculations

- What are the theoretical and practical limitations of Nortan’s Theorem?

Limitations of Norton’s Theorem include

Applicable only to linear and bilateral circuits

Cannot be directly applied to non-linear elements like diodes and transistors

Difficult to use in circuits containing only dependent sources without modification

Circuit simplification may become lengthy for large networks

Norton equivalent changes when circuit parameters change

aPractical measurements may contain errors due to component tolerances