Circuit Analysis (Lab 7)

Experiment 7: Analysis of Operational Amplifiers Operations

RABEEA IMTIAZ	2025-CE-75

Name: …………………………….	Registration No: …………………

Date:  …………………………….	Grade and Signature: …………………

| CLO4 | Construct amplifiers with operational amplifiers. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/ Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

# Objective

The objective of this lab is to investigate the input to output relationship for inverting and non- inverting amplifiers.

# Apparatus Required

| Sr. No. | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 1k Ω, 2k Ω ,3k Ω | 3 |

Description

Operational amplifiers can be used in two basic configurations to create amplifier circuits. One is the inverting amplifier where the output is the inverse or 180° out of phase with the input, and the other is the non-inverting amplifier where the output is in the same sense or in phase with the input.

Both operational amplifier circuits are widely used and they find applications in different areas. When an operational amplifier or op-amp is used as a non-inverting amplifier it only requires a few additional components to create a working amplifier circuit.

Pin configuration of IC 741 op amp is shown below

- Pin 1 is offset null.

- Pin 2 is inverting input terminal.

- Pin 3 is a non-inverting input terminal.

- Pin 4 is negative voltage supply (VCC)

- Pin 5 is offset null.

- Pin 6 is the output voltage.

- Pin 7 is positive voltage supply (+VCC)

- Pin 8 has no connection.

## Precautions:

- Voltage control knob should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position.

- Carefully utilize the oscilloscope channels and function generator.

## Procedure for Inverting Amplifier:

- Wire up the circuit as shown in the figure below on the breadboard.

- Give the input to the inverting terminal of the op-amp and simultaneously display the input and output on the oscilloscope.

- Draw the input & output waveforms at the end of the lab.

## Procedure for Non-Inverting Amplifier:

- Wire up the circuit as shown in the figure below on the breadboard.

- Give the input to the non-inverting terminal of the op-amp and simultaneously display the input and output on the oscilloscope.

- Draw the input & output waveform at the end of the lab.

## Result:

Figure 7: Input Voltage Signal	Figure 8: Inverted Output

Figure 9: Non-Inverting Output

# Post Lab Questions

- Define inverting amplifier.

  An inverting amplifier is an op-amp circuit in which the input signal is applied to the inverting     (−) terminal.

  The output signal is 180° out of phase (inverted) with respect to the input.

  It uses a feedback resistor to control gain.

2.

3.

- Define Non-inverting amplifier.

- A non-inverting amplifier is an op-amp circuit where the input is applied to the non-inverting (+) terminal.

- The output signal is in phase with the input (no inversion).

- It provides high input impedance and stable gain.

4.

5.

6.

- What are the applications of inverting amplifiers?

-   Used in signal inversion (phase reversal circuits).

-   Used in summing amplifiers (adding multiple inputs).

-   Used in integrator and differentiator circuits.

7.

8.

9.

- What are the applications of non-inverting amplifiers?

-   Used as voltage followers (buffer circuits).

-   Used in audio amplifiers for signal amplification.

-   Used in sensor signal conditioning circuits.

10.

11.

12.

- What is gain of an amplifier?

  Gain is the ratio of output signal to input signal.

  It shows how much the amplifier increases the signal strength.

-   It can be expressed as voltage gain, current gain, or power gain.

13.

14.

15.

- How do you calculate the gains for inverting and non-inverting amplifiers?

- Gain of inverting amplifier:

Av=−RfRinA_v = -\frac{R_f}{R_{in}}Av​=−Rin​Rf​​

- Gain of non-inverting amplifier:

Av=1+RfRinA_v = 1 + \frac{R_f}{R_{in}}Av​=1+Rin​Rf​​

- Where RfR_fRf​ is feedback resistor and RinR_{in}Rin​ is input resistor

16.

17.

18.