TASK 1

Lab Report: Series AC Circuit Simulation in Proteus

1. Objective
The objective of this lab is to design and simulate a series AC circuit using Proteus Design Suite and to measure the voltage across each resistor and the total current flowing through the circuit.

2. Components Required

- AC Voltage Source (VSINE): 5 V amplitude, 50 Hz frequency

- Resistor R1: 5.1 kΩ

- Resistor R2: 3.3 kΩ

- AC Voltmeter (2 units)

- AC Ammeter (1 unit)

- Ground terminal

3. Procedure

Opening Proteus:
Opened Proteus 8 Professional and selected “Schematic Capture”.

Component Selection:
Used the Pick Device (P) tool to select VSINE and resistors.

Circuit Assembly:

- Placed AC source (V1)

- Connected R1 and R2 in series

- Connected the circuit to ground

Parameter Configuration:

- Set source amplitude = 5V

- Set frequency = 50 Hz

- Set R1 = 5.1 kΩ

- Set R2 = 3.3 kΩ

Instrument Connection:

- Connected voltmeters across R1 and R2 (parallel)

- Connected ammeter in series

Simulation:
Ran the simulation and noted readings.

4. Observations & Results

Voltage across R1 = 3.03 V
Voltage across R2 = 1.96 V
Total Current = 0.59 mA

5. Calculations (Verification)

Total Resistance:
RT = R1 + R2
RT = 5.1k + 3.3k
RT = 8.4 kΩ

Total Current:
I = V / RT
I = 5 / 8.4k
I = 0.595 mA

Voltage across R1:
VR1 = I × R1
VR1 = 0.595 mA × 5.1k
VR1 = 3.03 V

Voltage across R2:
VR2 = I × R2
VR2 = 0.595 mA × 3.3k
VR2 = 1.96 V

6. Conclusion

The simulation was successful. It is verified that in a series circuit, current remains the same throughout the circuit, while voltage divides across resistors according to their values. The practical results closely match the theoretical values

# TASK 2

# Lab Report: DC Parallel Circuit Analysis

## 1. Objective

The objective of this experiment was to design and simulate a DC parallel circuit using Proteus 8 Professional. The aim was to measure the currents through three resistors (10kΩ, 2kΩ, 1kΩ) and verify that voltage remains the same across all branches.

## 2. Apparatus / Tools

Software:
Proteus 8 Professional

Components:

- 9V DC Battery (VSOURCE)

- Resistors: 10kΩ, 2kΩ, 1kΩ

- DC Ammeters (mA range)

- DC Voltmeter

- Connecting wires

## 3. Procedure

- Open Proteus 8 Professional and create a new project.

- Click on “Pick Devices (P)” and select battery and resistors.

- Place the 9V battery on the left side.

- Place three resistors in parallel.

- Connect all top terminals to positive and bottom terminals to negative.

- Add ammeters in series with each resistor.

- Set ammeters to milliamp (mA) range.

- Connect a voltmeter across the parallel network.

- Run the simulation and note readings.

## 4. Observations

Current through R1 (10kΩ) = 0.90 mA
Current through R2 (2kΩ) = 4.50 mA
Current through R3 (1kΩ) = 9.00 mA
Voltage across circuit = 9 V

## 5. Theoretical Calculations

Formula:
I = V / R

For R1 (10kΩ):
I1 = 9 / 10000
I1 = 0.0009 A
I1 = 0.9 mA

For R2 (2kΩ):
I2 = 9 / 2000
I2 = 0.0045 A
I2 = 4.5 mA

For R3 (1kΩ):
I3 = 9 / 1000
I3 = 0.009 A
I3 = 9.0 mA

Voltage in parallel circuit:
Vtotal = V1 = V2 = V3 = 9 V

## 6. Conclusion

This experiment shows that in a DC parallel circuit, voltage remains the same across all branches while current divides according to resistance. The simulated results match the theoretical calculations, proving the circuit is correct.