# 📘 LAB REPORT: THEVENIN’S THEOREM

## 🧪 Experiment Title

Verification of Thevenin’s Theorem using DC Circuit in Proteus

## 🎯 Objective

- To verify

- To find:

  - Thevenin Voltage (VTH)

  - Thevenin Resistance (RTH)

- To compare original circuit current and equivalent circuit current

## 🧰 Apparatus / Software

- Proteus Design Suite

- DC Voltage Source (VDC)

- Resistors (330Ω ×3, 1kΩ ×1)

- DC Ammeter

- Voltmeter

- Ground

## 📊 Circuit Description

Circuit consists of:

- 3 × 330Ω resistors (network formation)

- 1 × 1kΩ load resistor

- DC source (10V)

- Ammeter in series with load

## ⚙️ Theory

states that:

Any linear circuit can be replaced by:

- A single voltage source (VTH)

- In series with a resistance (RTH)

## 🧪 Procedure

### 1. Load Current Circuit

  Build the circuit in Proteus as given in diagram

  Connect ammeter in series with 1kΩ load

  Apply DC voltage (10V)

###   Note the load current2. Find VTH (Open Circuit Voltage)

  Remove the load resistor (1kΩ)

  Open the output terminals

  Connect a voltmeter across terminals A and B

###   Record the voltage → VTH

### 3. Find RTH

  Turn off the voltage source (replace with short circuit)

  Measure resistance from output terminals

###   Record value → RTH

### 4. Thevenin Equivalent Circuit

- Replace original network with:

  - VTH voltage source

  - Series resistance RTH

- Reconnect 1kΩ load

- Measure load current again

## 🧾 Observations

### Given / Measured Values:

| Quantity | Value |
| --- | --- |
| Supply Voltage (E) | 10V |
| VTH | 5.0V |
| RTH | 495Ω |
| Load Current (Original Circuit) | 3.34 mA |
| Load Current (Equivalent Circuit) | 3.33 mA |

## 📐 Sample Calculation

RTH=330+330×330330+330R_{TH}=330+\frac{330\times330}{330+330}RTH​=330+330+330330×330​

RTH=330+165=495 ΩR_{TH}=330+165=495\,\OmegaRTH​=330+165=495Ω

## 📈 Result / Verification

- Original circuit current = 3.34 mA

- Thevenin equivalent current = 3.33 mA

👉 Almost same values → Thevenin’s theorem verified ✔️

## ⚠️ Precautions

- Ground must be properly connected

- Ammeter always in series

- VDC must be OFF while finding RTH

- Connections should match diagram exactly