## Circuit Analysis (Lab # 6)

Experiment 6: VERIFICATION OF KIRCHHOFFS LAWS

RABEEA IMTIAZ	      2025 –CE-75

#### Name: …………………………….		Registration No:……………………. Date:  …24-4-26………………………….	Grade and Signature: ………………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeters, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

# Objective

To verify Kirchhoff’s current law and Kirchhoff’s voltage law for the given Circuit.

Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 330ohm, 220ohm 1kohm | 3 |

### Description

KCL: The algebraic sum of the currents meeting at a node is equal to zero.

KVL: In any closed path / mesh, the algebraic sum of all the voltages is zero.

### Precautions:

- Voltage control knob should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position.

### Procedure for KCL:

- Give the connections as per the circuit diagram.

- Set a value in RPS.

- Note down the corresponding readings using multimeter

- Repeat the same for different voltages

### Circuit - KCL

### KCL - Practical Values:

| Sl. No. | Voltage E | Current |  |  | I1 = I2 + I3 |
| --- | --- | --- | --- | --- | --- |
|  |  | I1 | I2 | I3 |  |
|  | Volts | mA | mA | mA | mA |
| 1 | 5 | 5.68 | 3.12 | 2.56 | 5.68=5.68 |
| 2 | 15 | 17.05 | 9.36 | 7.69 | 17.05=17.05 |
| 3 | 25 | 28.40 | 15.60 | 12.80 | 28.04=28.04 |

### Procedure for KVL:

- Give the connections as per the circuit diagram.

- Set a value in RPS.

- Note all the voltage reading

- Repeat the same for different voltages

### Circuit - KVL

### KVL - Practical Values

| Sl. No. | RPS | Voltage |  |  | E1 = V1 + V2 |
| --- | --- | --- | --- | --- | --- |
|  | E1 | V1 | V2 | V3 |  |
|  | V | V | V | V | V |
| 1 | 5 | 3.23V | 1.77V | 1.77V | 5=3.23+1.77=5 |
| 2 | 10 | 6.47V | 3.53V | 3.53V | 10=6.47+3.53=10 |
| 3 | 15 | 9.70V | 5.30V | 5.30V | 15=9.70+ |

### Result:

Thus, Kirchhoff’s voltage law and Kirchhoff’s current law are verified practically by performing this experiment.

### Post Lab Questions

##### 1.Define Kirchhoff’s current law.

“At any node (junction) in an electrical circuit, the sum of currents entering the node is                       equal    to the sum of currents leaving the node.”

Mathematically:

##### ∑Iin=∑Iout ​

##### 2.Define Kirchhoff’s voltage law.

“In any closed loop of a circuit, the algebraic sum of all voltages is zero.”

Mathematically:

∑V=0

OR:

##### E=V1+V2+V3

##### 3.What are the applications of regulated power supply?

A regulated power supply is used where constant voltage is required. Its main applications are:

- Electronic circuits and devices

- Computers and communication systems

- Medical equipment

- Laboratory experiments

- Industrial control systems

- Microcontrollers and embedded systems

##### 4.What are the limitations of KCL?

The limitations of Kirchhoff’s Current Law are:

1.Assumes no charge storage at the node (ideal condition)

2.Not directly applicable at very high frequencies (due to capacitance effects)

3.Does not consider electromagnetic radiation effects

4.Assumes lumped circuit elements only

5.Can be difficult to apply in complex 3D circuits with distributed elements.