Submitted By:

RABEEA IMTIAZ – 2025-CE-75

Assignment:

Lab No. 1

Submitted To:

Sir Mehmood

For the fulfillment of,

Circuit  Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

Post Lab Questions

### 1. Define Oscilloscope and write its applications.

An oscilloscope is an electronic instrument used to display and analyze electrical signals in the form of waveforms. It shows how voltage changes with respect to time on a screen.

Applications:

- To measure voltage, frequency, and time period of signals.

- To observe waveforms such as sine, square, and triangular waves.

- To detect noise or distortion in electronic circuits.

- To test and troubleshoot electronic devices and communication systems.

### 2. What is Peak to Peak Voltage?

Peak-to-Peak Voltage (Vpp) is the total voltage difference between the maximum positive peak and the maximum negative peak of a waveform.

It represents the full amplitude of the signal from the highest point to the lowest point.

### 3. What is RMS Value?

The RMS (Root Mean Square) value of a waveform is the effective value of an alternating voltage or current.

It is equal to the DC value that would produce the same power in a resistor as the AC signal.

### 4. Write the RMS values of Sine, Square and Triangular wave

The RMS values of different waveforms are:

- Sine Wave:
Vrms = Vp / √2 (approximately 0.707 × Vp)

- Square Wave:
Vrms = Vp

- Triangular Wave:
Vrms = Vp / √3 (approximately 0.577 × Vp)

Where Vp represents the peak voltage of the waveform.

### 5. Explain DC Offset

DC offset is the amount by which an AC signal is shifted above or below the zero voltage level.

If a waveform is not centered around zero volts, it means a DC component is added to the AC signal, causing the whole waveform to move upward or downward on the oscilloscope display.

For example, if a sine wave is shifted upward by 2 V, then the DC offset is +2 V.